General Design Guidelines

General

- Trace to trace spacing digital to digital – 5H and at least 3H if you get space issues.

- Trace to Trace spacing - digital to RF – use 20H separation between RF and digital signals and at least 10H if you get space issues.

- Do not split RF and digital grounds to avoid EMI issues, just keep the 20H separation rule above.

Power and Decupling

- Place small decupling first close to the power pin, then medium size and then bulk capacitors.

- Power traces/polygons - Route the power pins with polygons to the capacitors as wide as possible and min power trace width of 0.3mm trace -> 1.7[mOhm/mm] -> 1A (PCB Trace width Calculator)

- LDO Power traces should be able to provide 2A bursts so keep power traces from the LDO with wide polygons and minimum power trace of 1mm trace -> 0.8[mOhm/mm] -> 2A (PCB Trace width Calculator)

- PIMIC/LDO input/output caps with small loops and close to the BUCK

- Use reference/stitching vias close to the POWER VIAS

- Keep small loops connection (vias placement tight)

- Keep feedback lines of the buck and the PIMIC away from circuitry.

- Add as many GND vias as possible for better EMI and heat dissipation.

- Switch note only as large as needed for current – (Keep the connection from the buck to the inductor wide enough for the current requirement but not more than that). (max width calculation needed here)

- Place ESD TVS diods as close as possible to the USB connectors.

- PowerGood and Enable signals are less critical and route it last(before stitching vias)

- Power vias: vias comes in pairs, move vias closer to reduce inductance for smaller loops.

- Split the power plane 4 to the zync voltages.

- Use L8 power polygons for the GSM Power

- Use chassis capacitor and connect all external connectors to the chassis capacitor.

Zync Routing

Use “dog-bone” method and fit the trace width dog bone as required for the controlled impedance traces (use the width from the width table)

XTAL - critical line

- Placement with no stubs of CL1/2 and R – see example below.

- Keep away from other High speed or PWM signals.

- Keep space as possible from the consumer input and outputs.

- Keep out from other high speed or critical lines.

- Note this design guide document: XTAL Design guidelines by ABARCON

- Example for XTAL layout and routing:

CMOS Oscillator Layout and Routing – Critical line

  - Place OSC as close as possible to its consumer and in the resistor.

  - Resistor right after and then short trace to the consumer

  - Place it in a way that with his resistor there are no stubs.

  - Decup oscillator below with short loops

  - Oscillator Layout Layout example:

EEPROM Layout and Routing

- The EEPROM is not very fast but still try to keep traces short.

- Option to place it in the bottom Layer and then:

  - Decupling close with short loops

  - Pull ups close.

- Example from previous project:

EMMC layout and routing (0.5mm pitch)

- See the attached document “Recommended_PCB_Routing_Guidelines_for_SHM_e.MMC.pdf”

- EMMC is with 0.5mm pitch but most of the pins are NC so can “de-populate” unused pins

- Placement close, up-right to the Zynq(SDIO pins there in upper-left)

- Terminations (pull ups) at the top layer near the EMMC

- Route SDIO traces:

  - to Layer 6 (good for high speed)

  - 50 ohm controlled impedance

  - Traces lengths as short as possible (about 30ps??)

  - Keep spacing between traces - more than the manufacturer limit(as possible)

For example:

Boot parts guidelines:

- QSPI Routing:

  - Data lines delay matching

  - QSPI CLK, a bit longer then the QSPI Data lines(and not shorter)

- JTAG Pins as 50 ohms (optional at layer 6 if there is enough space)

- Placement:

  - Place dip switch on the bottom (trough hole VIA) to avoid stubs

  - Place dip switch resistors (3 pull ups and 3 pull downs) near the dip switch on the bottom

  - Place the JTAG header(not used in this project) above the DIP switch on the TOP(avoid stubs)

Differential lines

- More important so first - Intra – pair skew - P to N matching (same diff channel)

- Then - Inter pair skew matching (diff to diff)

GSM part

- Placement on the bottom and route to the power planes on layer

- Keep spacing of 20H of RF signals(GSM and muRata) to digital signals

- Use the placement recommendation drawing in the SARA-G450 - System integration manual for example:

DDR3L Memory:

- DDR Routing – Zync - > DDR3L -> VTT Resistors -> bypass capacitors

- Route clock signals first

- DDR3 DQ/DM to DQS_P/N skew limit is 50ps maximum but keep as low as possible.

- DDR3 Address to Clock Skew Limit is 103ps Maximum but keep as low as possible.

- DDR Traces Length - as short as possible and 8.55” (22cm) Maximum(UG933)

- VTT resistor slightly after the DDR3L RX (and less than a TL length)

- Trace to Trace– spread above the manufacturer limit when space allows.

- DDR differential traces guidelines for 80[ohm]±10% controlled diff Impedance:

  - See controlled impedance table

- Length matching/Delay matching  +-10ps (UG988 table-5-9)

- Via stub Spacing

- Suggested routing layer – layers 6 and 8(8 is preferred due to less stub):

| Layer | Type |
| --- | --- |
| L1 | Signal |
| L2 | GND |
| L3 | POWER |
| L4 | Signal (low speed, PMODS) + Power |
| L5 | GND |
| L6 | Signal (High speed) |
| L7 | GND |
| L8 DDR | Signal (High speed, less Stub) + Power |
| L9 | GND |
| L10 | Signal |

- Layout the following groups with same routing/layout/layer/vias:

  - ACC_Group:

    - Address lines [0-14]

    - BA[0-2]

    - CK_P, CK_N, CLKE

    - NCS, RAS,

    - CAS

  - Data_Group0 – DDR BL0:

    - DQ[0-7]

    - DDR_DM0

    - DDR_DQS0_N

    - DDR_DQS0_P

  - Data_Group1 – DDR BL1:

    - DQ[8-15]

    - DDR_DM1

    - DDR_DQS1_N

    - DDR_DQS1_P

- Note 1 - DQ Only - Bytes groups swapping is optional on the Memory device size.

- Note 2 - DQ Only - bits in Bytes groups swapping is optional on the Memory device size.

USB and USB PHY

Route USB to impedance 0f 90 ohm

Impedance calculators

https://www.pcbway.com/pcb_prototype/impedance_calculator.html

https://www.eeweb.com/tools/microstrip/

Controlled impedance table

| Controlled impedance all units in [mm] | Trace Thickness T | substrate height H | trace width W | Spacing S | Substrate Dielectric Er |
| --- | --- | --- | --- | --- | --- |
| 50 ohm SE microstrip | 0.035 | 0.11 | 0.2 | - | 4 |  |
| 50 ohm SE stripline | 0.035 | 0.11 | TBD | - | 4.29 |  |
| 40 ohm SE microstrip | 0.035 | 0.11 | 0.29 | - | 4 |  |
| 40 ohm SE stripline | 0.035 | 0.11 | TBD | - | 4.29 |  |
| 80 ohm diff microstrip | 0.035 | 0.11 | 0.13 | 0.2 | 4.9 |  |
| 80 ohm diff Stripline | 0.035 | 0.11 | 0.3 | 0.2 | 4 |  |
| 90 ohm diff(USB) - microstrip | 0.035 | 0.11 | 0.3 | 0.2 | 4.9 |  |
| 90 ohm diff(USB) - stripline | 0.035 | 0.11 | TBD | TBD | 4.29 |  |
| 100 ohm diff microstrip | 0.035 | 0.11 | 0.21 | 0.2 | 4.29 |  |
| 100 ohm diff stripline | 0.035 | 0.11 | TBD | TBD | 4.29 |  |

microstrip trace to SE 50ohm:

Top trace to SE 40ohm:

Internal strip line dimensions diff 80 ohms (DDR)

microstrip dimensions diff 80 ohms (DDR)

Internal strip line dimensions diff 90 ohms (USB)

Differential 100  ohm